HQ Main Network: 10.0.0.0 /16

| LAN | VLAN | Floor no. | Network address | Prefix | First address | Last address | Broadcast address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Accounting & Finance | 10 | 2 | 10.0.10.0 | /24 | 10.0.10.1 | 10.0.10.254 | 10.0.10.255 |
| Sales & BD | 20 | 1 | 10.0.20.0 | /24 | 10.0.20.1 | 10.0.20.254 | 10.0.20.255 |
| Executive & management | 30 | 3 | 10.0.30.0 | /24 | 10.0.30.1 | 10.0.30.254 | 10.0.30.255 |
| IT | 40 | 1 | 10.0.40.0 | /24 | 10.0.40.1 | 10.0.40.254 | 10.0.40.255 |
| Servers | 50 | - | 10.0.50.0 | /28 | 10.0.50.1 | 10.0.50.14 | 10.0.50.15 |
| DMZ | 60 | - | 10.0.60.0 | /28 | 10.0.60.1 | 10.0.60.14 | 10.0.60.15 |

Branch 1 Network 10.1.10.0/24

| LAN | VLAN | Floor no. | Network address | Prefix | First address | Last address | Broadcast address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Users | 10 | 1 | 10.1.10.0 | /24 | 10.1.10.1 | 10.1.10.254 | 10.1.10.255 |
|  |  | 2 |  |  |  |  |  |

| LAN | VLAN | Floor no. | Network address | Prefix | First address | Last address | Broadcast address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Users | 10 | 1 | 10.2.10.0 | /24 | 10.2.10.1 | 10.2.10.254 | 10.2.10.255 |
|  |  | 2 |  |  |  |  |  |

Branch 2 Network 10.2.10.0/24

| Subnet | Ongoing interface | Outgoing interface | Link type | Network address | Prefix | Description |
| --- | --- | --- | --- | --- | --- | --- |
| HQ | E0/1 | E0/0 | LAN P2P | 10.1.1.0 | /30 | HQ-Core to HQ-Dest1 |
|  | E0/0 | E1/1 | LAN P2P | 10.1.1.4 | /30 | HQ-Core to HQ-Dest2 |
|  | E0/2 | Port3 | LAN P2P | 10.1.2.0 | /30 | HQ-Core to HQ-FGT |
|  | Port2 | Gi1 | WAN P2P | 192.168.1.0 | /30 | HQ-FGT to HQ-EDGE-R1 |
| B1 | E0/2 | Port3 | LAN P2P | 10.2.2.0 | /30 | B1-Core to B1-FGT |
|  | Port2 | Gi2 | WAN P2P | 192.168.2.0 | /30 | B1-FGT to B1-EDGE-R1 |
| B2 | E0/2 | Port3 | LAN P2P | 10.2.1.0 | /30 | B2-Core to B2-FGT |
|  | Port2 | Gi2 | WAN P2P | 192.168.3.0 | /30 | B2-FGT to B2-EDGE-R1 |
| ISP | Gi1 | Gi2 | WAN P2P | 1.1.1.0 | /30 | ISP to HQ-EDGE-R1 |
|  | Gi2 | Gi1 | WAN P2P | 2.2.2.0 | /30 | ISP to B1-EDGE-R1 |
|  | Gi2 | Gi1 | WAN P2P | 3.3.3.0 | /30 | ISP to B2-EDGE-R1 |

Point-to-Point Subnets